Gun Upgrades for SAD Aug – Sept 2019

I have listed the four main upgrades planned for the summer SAD.   With each I have listed some objectives and considerations.   Give some thought, we’ll meet for feedback and decisions.

The four main upgrades…

Upgrade to 200kV cathode electrode

- Test new small-shed cathode electrode at UITF to 200kV and for beam quality

- Build-up a new black insulator w/ degassed new style flange

- After UITF testing, move cathode electrode to new flange/insulator

- Pre-survey, ready for installation at CEBAF

Upgrade anode feedthrough voltage capability

- While gun is vented, replace 600V BNC feedthrough w/ 10kV SHV feedthrough

- New anode bias supply (+2.5kV) or standard UHV bias supply (+6.5kV) are ready

Upgrade krypton plumbing line configuration

- Matt installed new Kr bottle, regulator, pump-out port like in fixed/convenient location

- Replace long bellows w/ steel tubing

- Simplify valves on gun flange e.g. make similar to UITF configuration?

Upgrade puck exchange functionality

- Replace “tee” w/ new load chamber to exchange two pucks at gun w/ one bake out

  - Modify the 4 puck holder w/ a 2 puck holder, mate with existing suitcase fork

  - Implement a vertical goes up/down linear actuator

  - Can we forego puck holder heating, agree to suitably degas pucks in advance?

  - Permanent heat tapes, thermocouple for reliability

  - Ion pump w/ overboard board for bakeout, good isolation valves

- Retain capability for suitcase to exchange four pucks w/ existing docking chamber

  - Replace leaky suitcase valve w/ new one (Marcy purchased already?)

  - We may need a longer manipulator to reach PREP, that’s OK

- Install improved permanent stand for load chamber and suitcase, w/ better alignment

“Old hotness” (left) at CEBAF vs. “new hotness” coming from UITF:

Kr plumbing on gun chamber at CEBAF (left) vs. UITF (right):

Kr bottle and supply line at CEBAF (left) vs. UITF (right):