From Joe 10/27/20 discussion

List of topics to think about:

- Assigning spin component information to a particle (or macroparticle)

- Implementing the Bargman-Michel-Telegdi time-dependent spin-equation of motion

- Generally rate of spin motion (ds/dt) is similar to physical motion (dx/dt), but maybe consider how time steps are computed, e.g. when an adaptive time step is implemented

- Implement methods to define initial spin distribution a) internally  (e.g. set sx,sy,sz of particles very useful for testing) or b) externally (e.g. to accept a distribution originating from another code like Geant4 output)

- Evaluate numerical test cases of 'pure fields' (e.g. B_perp, B_axial, E_perp, E_axial) or single elements (e.g. dipole, solenoid, e-field deflector) against analytic solutions

- Evaluate numerical test case of field map (e.g. we have ExB wien filter field maps, solenoids, cavity)

- Saving the spin component information, similar to other arrays, in output file

- Ability to display spin component (or polarization = average of sx, sy or sz) in GUI would be useful

From Bas 11/9/20 discussion

Let’s already try to agree on a few things (very important technical details):

- The spin coordiantes will be called ’sx’, ’sy’ and ’sz’. This will be a real pain if we need to change this later. Choose wisely.

- They can be read from file using (to be modified) setfile if the corresponding columns are available

- You let me know what other methods you would like to see for initialization. This is NOT trivial.

- Output will be with the same ’sx’, ’sy’ and ’sz’ columns if you agree with the name

- I create the differential equations for spin, but I am not sure if they will slowly diverge and get a length != 1. Maybe we have to (re)-normalise them after each timestep. No big deal, also not at all a performance issue, but we should not forget about this.

- Display is already done since I programmed a GDF to ParaView converted and this allows you to for example plot particle coordinates as spheres, velocity vectors as 3D arrows and 3D spin vectors as whatever 3D shape/color you like.