Laboratory Directed Research and Development Proposal
Title:  Generation and Characterization of Magnetized Bunched Electron Beam from DC Photogun for MEIC Cooler

| Lead Scientist or Engineer: | RiaD Suleiman and Matt Poelker |
| --- | --- |
| Phone: | (757) 269-77159 |
| Email: | suleiman@jlab.org |
| Date: | April 20, 2015 |
| Department/Division: | Center for Injectors and Sources / Accelerator Division |
| Other Personnel: |  |
| Proposal Term: | From: 10/2015 Through: 10/2017 If continuation, indicate year (2nd/3rd): |

| Division Budget Analyst | Kelly Webster |
| --- | --- |
| Phone: | (757) 269-7575 |
| Email: | hanifan@jlab.org |

This document and the material and data contained herein were developed under the sponsorship of the United States Government. Neither the United States nor the Department of Energy, nor the Thomas Jefferson National Accelerator Facility, nor their employees, makes any warranty, express or implied, or assumes any liability or responsibility for accuracy, completeness or usefulness of any information, apparatus, product or process disclosed, or represents that its use will not infringe privately owned rights. Mention of any product, its manufacturer, or suppliers shall not, nor it is intended to imply approval, disapproval, or fitness for any particular use. A royalty-free, non-exclusive right to use and disseminate same for any purpose whatsoever, is expressly reserved to the United States and the Thomas Jefferson National Accelerator Facility.

Abstract

This LDRD proposal aims to generate magnetized, rf-bunched electron beam from a DC high voltage photogun.  Measurements of beam magnetization as a function of laser spot size and magnetic field strength at photocathode are planned. Round-to-flat beam transformation will be performed using three skew quadrupoles and the transverse emittance ratios will be measured. Photocathode lifetime at milli-ampere currents will be compared to beam lifetimes with no magnetization, to study the effect of the solenoid field on photocathode ion-back bombardment. Afterwards, a follow-up proposal can be submitted to evaluate the merits of magnetized beam generation using a DC high voltage thermionic gun, with rf-pulsed gridded thermionic emitter.  Combined, these measurements will provide insights on ways to optimize the MEIC electron cooler, and help us choose the appropriate electron source.

# Summary of Proposal

## Description of Project

To achieve the required luminosity, ion beams at MEIC must be cooled.  In general, this is accomplished when an electron beam co-propagates with an ion beam moving at the same average velocity (γe=γi), but different temperatures (Te≪Ti), where the energy of chaotic motion of the ion beam is transferred to the cold electron beam. As proposed by Derbenev [1], the cooling rate can be improved by about two orders of magnitude if the process occurs inside a solenoidal field that forces the electrons to follow small helical trajectories thereby increasing the interaction time with ions and improving the cooling efficiency. This cyclotron motion also provides suppression of electron-ion recombination. Cooling rates with magnetized electron beam are ultimately determined by electron longitudinal energy spread rather than the electron beam transverse emittance.

The envisioned MEIC magnetized cooler is part of the Collider ring and aims to counteract emittance degradation induced by intra-beam scattering (IBS), to maintain emittance during collisions and extend the luminosity lifetime. To implement cooling at relatively high energy (electron beam energy 55 MeV (γ=108)), the electron beam must be bunched and accelerated in an SRF linac. The MEIC cooling solenoid is 30 m long providing a 2 T field. Table 1 summarizes the requirements on the electron gun with noteworthy challenges related to bunch charge and average current, 420 pC and 200 mA, respectively.

Table 1: Requirements on bunched electrom beam for MEIC magnetized cooling

| Bunch length | 100 ps (3 cm) |
| --- | --- |
| Repetition rate | 476 MHz |
| Bunch charge | 420 pC |
| Peak current | 4.2 A |
| Average current | 200 mA |
| Beam radius at cathode | 3 mm |
| Transverse normalized emittance | 10s microns |
| Solenoid field at cathode | 2 kG |

One challenge associated with implementing cooling inside the long solenoid of the Collider, is the fringe field immediately upstream of the cooling solenoid.  The field lines outside the solenoid magnet introduce very large beam rotation. Derbenev [2] suggested the ill-effects of this fringe field could be cancelled if the electron beam was born in a similar field, but producing beam rotation in the opposite direction, such that the two cancel.

Although, electron cooling with DC electron beams at low energy has been implemented at many labs, no one has yet demonstrated electron cooling with bunched electron beams, or magnetized cooling. Fermi Lab successfully demonstrated non-magnetized relativistic DC cooling at high energy (4.3 MeV) [3].  For Low Energy RHIC Electron Cooling (LEReC) non-magnetized bunched electron beam will be used and eRHIC is planning to use Coherent Electron Cooling (CeC) [4].

There are four electron gun options to consider: SRF gun, Normal conducting RF gun (with a photocathode or gridded thermionic cathode), DC high voltage gun (with downstream bunching, or rf-pulsed gridded thermionic emitter), and a DC high voltage photogun.

In general, RF guns are more complicated and expensive compared to DC guns.    The biggest challenge for a normal conducting RF gun is thermal heat load management.   Typically, normal RF guns are used for pulsed-rf applications.   Groups have tried to operate normal conducting RF guns in CW mode but so far, with little success (Los Alamos), or with very low resultant beam energy, comparable to (or lower than) energy produced by DC high voltage photoguns (AES, FarTech).  SRF guns promise CW operation and with high average current and beam energy, but to date, no effort has come close to achieving the desired high average current (Rosendorf, BNL).   Moreover, no attempt has been made to produce magnetized beam from an SRF gun, because the application of a magnetic field on the photocathode is highly problematic to maintaining the superconducting condition of the SRF cavity due to the Meissner effect.

A DC high voltage rf-pulsed gridded thermionic gun, similar to that used at TRIUMF [5], is a viable option since the requirement on gun emittance is not stringent.  Of all electron gun options, thermionic guns are considered the closest to “turn-key” technology, relatively simple to operate and maintain, even at very high average current. However, the beam requirements of the Energy Recovery Linac (ERL) may necessitate a smaller emittance from the gun.

Thus we are left with the DC high voltage photogun option.  Compared to thermionic guns, photoguns allow for precise control of the electron bunch profile in space and time, which could help to increase the cooling efficiency. Two drawbacks of the dc high voltage photogun are field emission at high bias voltage and the reliance on a relatively delicate photocathode. The JLab gun group has made progress toward minimizing/eliminating field emission. In addition, the gun group has recently manufactured alkali-antimonide photocathodes, shown to be less sensitive to ion bombardment, and demonstrating long lifetime at milli-ampere currents.  Based on these considerations, and including our expertise with photoguns, a DC high voltage photogun is the focus of this LDRD proposal.

We also note that a follow-up to this proposal could be submitted to study magnetized beam created using a DC high voltage thermionic gun, with rf-pulsed gridded thermionic emitter.   We recently helped TRIUMF develop such a gun, by lending them our original 100 kV thermionic gun.   They modified this gun to include the application of rf to the bias grid, and successfully produced rf-bunched beam at high average current and 650 MHz repetition rate, with 100 ps pulses.   This gun could be returned to JLab for tests using the same diagnostic beamline and gun solenoid described below.

Examining Table 1, three challenging requirements can be identified. First, the average current is very large. We note that groups from other labs and universities are working hard to achieve high average current and high bunch charge for a variety of applications. The Cornell University group, for example, has recently delivered 65 mA for about 9 hours with lifetime of 2.6 days [6]. Key to this success was the re-discovery of alkali-antimonide photocathode, which is less sensitive to ion bombardment. At JLab, we have used the K2CsSb photocathode to demonstrate long-lifetime operation at 10 mA average current [7]. The second challenge relates to high bunch charge at high repetition rate.   For a DC high voltage gun, this means the gun must be biased at very high voltage, to create a “stiff” beam that is less susceptible to space charge force.   We have made relatively good progress developing a 350 kV gun.  This work is on-going. The final challenge is the generation and transportation of magnetized bunched electron beam from a gun. Although many labs are pursuing electron cooling, to the best of our knowledge, only one group has been studying magnetized beam: the Piot group at the Fermilab Photoinjector Laboratory [8,9]. However, they are not making CW beam at high average current – the new Fermi Lab ASTA injector relies on a pulsed NCRF gun with a Cs2Te photocathode illuminated by an ultraviolet (UV, λ=263 nm) laser pulse at 0.5% duty factor [10].  No outside groups appear to be working to develop high average current magnetized beam.  This is a very serious problem for the MEIC and the reason we assert this the proposed LDRD is essential for JLab.

The narrative of magnetized electron cooling is as follows (using cylindrical coordinates (r,ɸ,z) with the vector momentum written as: p=prr+pϕϕ+pzz). With Boltzmann Constant, kB =8.217x10-5 eV/K, mec2=510998 eV, c=299792458 m/s, e/m=1.759x1011 rad/(s T):

- Electrons are born in a uniform magnetic field Bcath = Bz = 0.2 T where the electron beam radius is a0 = 3.0 mm (this is the same as the laser spot size). The photocathode effective temperature is about Tc = 1000 K. For this cathode the thermal energy is kBTc = 0.086 eV, the transverse momentum is p⟘0=2mekBTc = 296.8 eV/c and the thermal emittance is ϵth=a0p⟘02mec=a0kBTc2mec2 = 0.87 µm.

- For a gun HV of 350 kV, β=0.8048, γ=1.685 and the momentum in the z direction is pz = βcγme = 693 keV/c.

- Since the electrons are born with transverse thermal momentum in magnetic field, each electron will have a helical (cyclotron) motion with radius rc=p⟘0eB = 4.95 µm. This cyclotron radius is very small when compared to the electron beam radius. The cyclotron frequency is ωc=eBzγme = 2.09x109 rad/s.

- The emittance (cyclotron emittance) due to cyclotron motion is: ϵc=kBTceBzc = 0.00144 µm; very small when compared to thermal emittance.

- After traveling a few centimeters from the photocathode, the electrons exit the longitudinal solenoid field but they encounter a returning solenoid radial field which exerts torque on the electrons that produces a rotating trajectory. This beam rotation must be tailored to cancel out similar motion encountered when entering the field of the cooling solenoid.

- Busch’s theorem represents the conservation of canonical angular momentum (CAM) and states that L=γmer2ϕ+e2πψ is a constant, where ϕ≡dϕdt is the angular velocity and ψ=B·dS is the magnetic ﬂux enclosed by the particle trajectory (i.e., the ﬂux inside a circle with radius r given by the radial distance r of the particle from the axis at a given position along the trajectory).

- From Busch’s theorem, at the photocathode, L=e2πψ and ψ=B·dS=πr2Bz, that is for an electron L=12eBzr2. And after leaving the solenoid L=γmer2ϕ. For cylindrically symmetric gaussian beam with rms size of a0, r2=2a02 and the average canonical angular momentum for the electron beam is L=eBza02 = 1800 (neV s) at the photocathode and L=2γmea02ϕ = 1800 (neV s) after existing the solenoid.

- The average angular velocity is ϕ=ωL=eBz2γme =1.04x1010 rad/s (i.e., the beam rotates at the Larmor frequency, ωL=ωc2). The average mechanical momentum in the ɸ direction is pɸ=γmeaoϕ = 89.94 keV/c. The electron momentum now has a phi component in addition to the axial z-component.

- The average rotational kinetic energy is Tϕ=mec22+pϕ2c2-mec2= 7854.3 eV. This is small (2.24%) when compared to the maximum longitudinal kinetic energy of Tz=350000 eV. The total kinetic energy is still Tz+Tϕ=350000 eV (i.e., part of the longitudinal (axial) kinetic energy turns in rotational energy in the phi direction).

- The electron beam comes out of the cathode solenoid field rotating but once in field free region where there is no centripetal force, the beam starts expanding. However, the mechanical angular momentum is still conserved.

- Many focusing solenoids installed after the exit of the gun and throughout the injector prevent the magnetized beam from blowing up.

- After leaving the cathode solenoid, and due to beam rotation induced by radial field the magnetized beam will have an emittance (called drift or magnetized emittance) of ϵd=eBza022mec = 528 µm, or in general, ϵd[µm] ~ 29.3 [kG] σe2 [mm] 2.

- The cyclotron emittance and drift emittance are related as ϵth= ϵcϵd where the ratio is ϵdϵth=ϵdϵc=a0rc=606 .

- The magnetized beam is a coupled beam, and for transport and acceleration, it is easier if the beam is transformed to a flat beam. This can be done with three skew quads (aka, Round-to-Flat Beam (RTFB) Transformer). This transformation produces a beam with very large transverse emittance ratios. For round beam, ϵx=ϵy, in contrast to flat beam where ϵx≫ϵy .

- Bush's theorem states that CAM is preserved in any field with an axially-symmetric 4-vector potential. A quadrupole lattice does not do so; ultimately it can turn CAM to zero (in case when the Larmor emittance in solenoid is zero, i.e., practically very small compared with the drift one) and also restore it later when needed.

- The electron beam is accelerated in SRF linac to high energy.

- Before entering the cooling solenoid, the beam is transformed back to round beam (magnetized beam) with Flat-to-Round Beam (FTRB) Transformer that restores the initial magnetization.

- At the entrance of the cooling solenoid, the radial field exerts a torque (opposite to cathode solenoid torque) so the beam is not rotating any more given that the magnetic field in both solenoids is in the same direction. This can be achieved when BCatha02=Bcoolσe2, where Bcool = 2 T and electron beam radius in the cooling section is σe = 0.95 mm.

- Typical size of the ion beam in the cooling solenoid is 1 mm (?) with an initial emittance (before cooling) of ϵion=30 µm (?) when it enters the cooling solenoid.

- Once inside the cooling solenoid, the electrons only have a cyclotron motion with radius rc=p⟘0eB = 0.49 µm but no overall rotational motion. The cyclotron emittance due to this motion is ϵc=kBTceBcoolc = 0.000144 µm.

- If the beam is not magnetized from the gun, then the additional rotational motion when it enters the cooling solenoid will make it much hotter than the ion beam (ϵd = 528 µm). Instead the emittance will be very close the thermal emittance from the cathode – since the cyclotron emittance is very small (albeit with some degradation due to space charge emittance growth but this can be compensated with focusing solenoids in the beamline).

- Hence: if the beam is not magnetized from the gun then no cooling takes place in a solenoid. Furthermore, magnetic shielding is now needed in the cooling section; non-magnetized cooling has very strong dependence on relative angles between electrons and ions and strict control of remnant magnetic fields is required to minimize the transverse angular spread in the cooling section.

- In the cooling solenoid, the electron energy is 55 MeV, with β=0.999957 and γ=107.63.

- For magnetized beam in the cooling solenoid, the cyclotron frequency is ωc=eBcoolγme = 3.27x109 rad/s, or fc=ωc2π = 0.52 GHz. Over the length of the cooling solenoid of 30 m, the electron will finish 52 periods of its cyclotron rotations.

- The helical motion of the electrons in the magnetic field increases the electron-ion interaction length, thereby significantly improving the cooling efficiency. In particular, electron-ion collisions that occur over many cyclotron oscillations and at distances larger than the cyclotron radius are insensitive to the transverse velocity of the electrons. This cyclotron motion also provides suppression of electron-ion recombination.

- Electron-ion collisions are described by Rutherford’s scattering formula and is determined (and hence the cooling rate) by the relative particle velocities’ spread, Δ(ve-vᵢ) in the co-moving frame (v⟘= γcϕ, v‖= cΔγ/γ), where ϕ is the transverse angle deviations.

- In general, electron cooling is preferred at low energy. At higher energies the cooling rate picks up a factor of 1/γ2  due to Lorentz contraction and time dilation. In the electron-ion rest frame, the length of the cooling solenoid is only 30/γ=0.28 m. In addition, in their rest frame, the electron and ion stay together for only 30/(βcγ)=90/γ=0.84 ns.

- For non-magnetized beam, the cooling rate is λ∝1/ve⟘3 (?) while for magnetized beam, the cooling rare is λ∝1/(v-ve‖)2 (?). Electron beam energy spread in MEIC cooler is required to be Δγe/γe≤10-4.

- Cooling solenoid must have high parallelism of magnetic field lines, ΔB⟘Bz≈ϵionnβγβmatch≈10-4, where ϵionn=0.4 µm is the ions normalized emittance and  βmatch=0.18 m is the beta function in the cooling solenoid.

- Upon exiting the cooling solenoid, the beam once again starts rotating due to canonical angular momentum conservation according to Busch’s theorem.

- Another RTFB transformation is needed before entering the Energy Recovery Linac (ERL). The electron beam is then dumped at low energy.

This is the narrative of magnetized electron cooling. Figure 1 shows a depiction of the magnetized electron cooling process.

Figure 1: Depiction of magnetized electron beam generation, RTFB transformation and magnetized cooling in a solenoid.

## Expected Results

JLab will have direct experience magnetizing a high current electron beam.  We will learn how the applied magnetic field influences the photocathode lifetime.   We learn about challenges associated with round to flat beam transformations at high bunch charge.

# Proposal Narrative

## Purpose/Goals

The goal of this LDRD is to generate magnetized beam and measure its properties. The impact of the cathode solenoid on the operation of the photogun will be explored. These measurements will provide insights on ways to optimize the MEIC electron cooler, and help us design the appropriate electron source.

## Approach/Methods

We will use the gun and injector beamline that is being built now at the FEL Gun Test Stand (GTS) (layout can be found in first attachment) to carry out the proposed LDRD work. The gun is an inverted ceramic with a K2CsSb photocathode and a green 532 nm drive laser (with frequencies of 4.76 MHz to 476 MHz). We have two HV supplies: Spellman 225 kV and 32 mA and Glassman 550 kV and 5 mA (with SF6 system). Figure 2 shows the HV chamber. For the proposed LDRD work we need to design and build a solenoid to produce 2 kG at the photocathode with uniformity over the active area to be specified later. The proposed beamline is shown in Figure 3. We will follow the prescription outlined in Ref. [11] to design the three skew quadrupoles.

Figure 2: Photogun HV chamber with inverted ceramic and a ball cathode. A solenoid magnet will be installed around the chamber to generate uniform field at photocathode.

Figure 3: Proposed beamline to generate magnetized beam and measure beam magnetization. Three skew quadrupoles will be used as RTFB transformer. Few focusing solenoids will be installed between the exit of the gun and dump to prevent the magnetized beam from blowing up.

We are planning on these measurements:

- Generate magnetized beam and measure magnetization for different laser spot sizes, (a0 = 0.1 – 3 mm), bunch charges (1 – 500 pC), bunch lengths (50 – 150 ps) and solenoid fields (0 – 2 kG). The magnetization will be measured using Diagnostics Cross 1 and Diagnostic Cross 2 with the three skew quads off via

L=2pz σ1σ2sinϕD=eBza02

where σ1 is the beam radius measured at Diagnostic Cross 1 and σ2 is the beam radius measured at Diagnostic Cross 2. The drift between the two crosses is D and pz is the longitudinal momentum of the beam (for a gun HV of 350 kV, pz = βcγme = 693 keV/c). The angular rotation ϕ is measured from the beam image at Diagnostics Cross 2 when the multislit is inserted at Diagnostics Cross 1. Figure 4 shows an example of a mechanical measurement done at Fermilab.

Figure 4: Example of data set used for mechanical angular momentum measurement at Fermilab. Figure from Ref. [8].

- Use the three skew quads – RTFB Transformer proposed by Derbenev [12] – to generate a flat beam with transverse emittance ratios of:

ϵxnϵyn= ϵdϵc= rc2ao2=2βγLϵthn2≫1

where L=L/2pz.

The horizontal and vertical emittance will be measured using the slit method. Diagnostics Cross 2 will be equipped with a horizontal and vertical slits. The size of the emittance dominated beamlets passed through the slits will be measured, after a drift distance D, with a YAG viewer in Diagnostics Cross 3. Assume that the horizontal beam radius measured at Diagnostics Cross 2 is σ2h and the horizontal radius of the beamlet at Diagnostics Cross 3 is σ3h when a vertical slit is inserted at Diagnostics Cross 2, then the horizontal emittance is ϵxn=γσ2hσ3h/D. Similarly the vertical emittance can be measured using a horizontal slit.

- Generate very high currents magnetized beam and study beam transport and RTFB transformation versus electron bunch charge.

- Measure magnetized photocathode lifetime at high currents (up to 32 mA) and high voltages (200 – 350 kV).

- Study beam halo and beam loss versus magnetization. We can do that using three different measurements:

  - Use ion pumps equipped with very sensitive current readback to monitor vacuum.

  - Radiation will also be monitored using x-ray detectors placed around the gun and beamline.

  - We will measure the beam intercepted at the floating anode.

## Specific Location of Work

The work will be performed in the FEL Gun Test Stand. Figures 5 and 6 show the existing beamline. This beamline will be modified to install the three skew quadrupoles.

Figure 5: The existing beamline top view. There is no buncher installed; only a drift beamline, where the three skew quads will be installed.

Figure 6: The existing beamline between the gun and the dump. This beamline will be modified to install the three skew quadrupoles. The buncher will not be installed (orange object)

## Anticipated Outcomes/Results

We hope to demonstrate experimentally many aspects of the magnetized bunched electron beam for the MEIC cooler, with the notable exception of 200 mA average beam current.  We are limited by the in-house HV supplies to 32 mA.

# VITA (Lead Scientist)

MATTHEW POELKER

Thomas Jefferson National Accelerator Facility

Newport News, VA 23606

Office Phone: (757) 269-7357			e-mail: poelker@jlab.org

Rank: Senior Staff Scientist	DISCIPLINE: Polarized electrons,

Photoelectron guns, Lasers, Vacuum

- Professional Preparation

B.S., Engineering Physics, University of Illinois, Champaign/Urbana, IL, December 1983

M.S., Electrical Engineering, Northwestern University, Evanston, IL, June 1988

Ph.D., Electrical Engineering, Northwestern University, Evanston, IL, June 1992

Postdoctoral research associate, Physics Division, Argonne National Lab, January 1992 to May 1994

- Appointments

1994-present:	Staff Scientist and Group Leader Center for Injectors and Sources, Thomas Jefferson National Accelerator Facility

- Recent Journal Publications

- “Improving the Performance of Stainless-Steel DC High Voltage Photoelectron Gun Cathode Electrodes via Gas Conditioning with Helium or Krypton.” M. BastaniNejad, A. A. Elmustafa, E. Forman, J. Clark, S. Covert, J. Grames, J. Hansknecht, C. Hernandez-Garcia, M. Poelker, R. Suleiman, Nucl. Instr. and Meth. A 762 135 (2014).

- “Outgassing Rates of Identical Stainless Steel Chambers Subjected to Different Heat Treatments and with Different Coatings.” M. A. Mamun, P. A. Adderley, M. L. Stutzman, M. Poelker and A. A. Elmustafa, Journal of Vacuum Science and Technology A 32, 021604 (2014).

# “Measurement of Electron Beam Polarization from Unstrained GaAs via Two-Photon Photoemission.” J. L. McCarter, A. Afanasev, T. J. Gay, J. Hansknecht, A. Kechiantz, M. Poelker, Nucl. Instr. and Meth. A 738, 149 (2014).

- “A search for spin-polarized photoemission from GaAs using light with orbital angular momentum.” N. B. Clayburn, J. L. McCarter, J. M. Dreiling, M. Poelker, D. M. Ryan, T. J. Gay, Phys. Rev. B 87, 035204 (2013).

- Awards

- E. O. Lawrence Award Recipient,  2011

- Committees

- International Spin Physics Committee since 2012

RIAD SULEIMAN

Jefferson Laboratory, Suite 8

600 Kelvin Dr., Newport News, Virginia 23606

Work: (757) 269-7159

e-mail: suleiman@jlab.org

Employment

- JEFFERSON LABORATORY:

March 2012 - present. Staff Scientist III, Accelerator Division.

January 2007 - February 2012. Staff Scientist II, Accelerator Division.

- VIRGINIA POLYTECHNIC INSTITUTE AND STATE UNIVERSITY:

August 2005 - January 2007. Research Scientist.

August 2004 - July 2005. Postdoctoral Research Associate.

- MASSACHUSETTS INSTITUTE OF TECHNOLOGY:

July 2002 - July 2004. Senior Postdoctoral Research Associate at the Laboratory for Nuclear Science.

October 1999 - June 2002. Postdoctoral Research Associate at the Laboratory for Nuclear Science.

Education

- KENT STATE UNIVERSITY: September 1993 - October 1999.

Ph.D. student at the Center for Nuclear Research of the Physics Department.

Doctor of Philosophy degree in Physics awarded in December 1999.

Thesis project (Adviser: Professor Makis Petratos):

“Measurement of the Electric and Magnetic Elastic Structure Functions of the Deuteron at Large Momentum Transfers”.

- YARMOUK UNIVERSITY, JORDAN: February 1990 - May 1993.

Bachelor of Science degree in Physics, 1993.

Professional Affiliation

- American Physical Society

Recent Journal Publications

- “Measurement of parity violation in electron-quark scattering.” D. Wang et al., Nature 506, 67 (2014).

- “Charge Lifetime Measurements at High Average Current Using a K2CsSb Photocathode inside a DC High Voltage Photogun.” R. Mammei, R. Suleiman, et al., Phys. Rev. ST Accel. Beams, 16, 033401 (2013).

- “First Determination of the Weak Charge of the Proton.” D. Androic et al., Phys. Rev. Lett. 111, 141803 (2013).

# Budget Explanation

To be able to perform the proposed measurements, budget is required for the following procurements:

- Purchase solenoid magnet, or Helmholtz coil-pair, to provide 2 kG at cathode

- Three skew quadrupoles to be used as a round-to-flat beam transformer

- Components installed inside three diagnostics crosses:

  - (3) YAG crystal view “screens”

  - Cameras and switcher controls

  - Multi-slit, water cooled

  - Single Slits in both planes, water cooled

- Vacuum hardware: stepper motor controlled bellows, for the multislit and movable single slits for emittance measurements.  (note, we have the big ticket items in-house: gun, K2CsSb deposition chamber, valves, dump, ion pumps, differential pump can and NEG modules, we have 80/20 aluminum support structure)

- Laser/optics components to vary the dimensions of the laser spot at the photocathode

- 50% postdoc

Work performed outside the gun group will need to be paid for with additional budget:

- Gun magnet design

- Gun magnet installation

- Relocate the old ARC dipole power supply and provide 480V and LCW.

- Mechanical designer for skew quad magnets (Matt Marchlik)

- Mechanical designer for the slits (Matt Marchlik)

- ASTRA and GPT modeling (Fay Hannon)

The total cost for the first year is $383,000. For the gun magnet power supply, we will use an old ARC dipole magnet supply (450 A/150 V). For the second year, the total cost is $119,000.  (loaded, overhead added)

References

- “Magnetization effects in electron cooling.” Ya. Derbenev and A. Skrinsky, Fiz. Plazmy 4, 492 (1978) [Sov. J. Plasma Phys. 4, 273 (1978)].

- “A low emittance, flat-beam electron source for linear colliders.” R. Brinkmann, Y. Derbenev, and K. Flöttmann, Phys. Rev. ST Accel. Beams 4, 053501 (2001).

- “Experimental Demonstration of Relativistic Electron Cooling.” S. Nagaitsev et al., Phys. Rev. Lett. 96, 044801 (2006).

- “Cooling of High-Energy Hadron Beams.” M. Blaskiewicz, Annual Review of Nuclear and Particle Science 64, 299 (2014).

- “The TRIUMF-ARIEL RF Modulated Thermionic Electron Source.” F. Ames, Proc. Of EIC’14, Newport News, VA (2014).

- “Record high-average current from a high-brightness Photoinjector.” B. Dunham et al., Appl. Phys. Lett. 102, 034105 (2013).

- “Charge Lifetime Measurements at High Average Current Using a K2CsSb Photocathode inside a DC High Voltage Photogun.” R. Mammei et al., Phys. Rev. ST Accel. Beams 16, 033401 (2013)

- “Generation of angular-momentum-dominated electron beams from a Photoinjector.” Y.-E Sun et al., Phys. Rev. ST Accel. Beams 7, 123501 (2004).

- “Photoinjector generation of a flat electron beam with transverse emittance ratio of 100.” P. Piot et al., Phys. Rev. ST Accel. Beams 9, 031001 (2006).

- “The Advanced Superconducting Test Accelerator (ASTA) at Fermilab:

A User-Driven Facility Dedicated to Accelerator Science & Technology.” P. Piot et al., arXiv:1304.0311 [physics.acc-ph] (2013).

- “Simple algorithm for designing skew-quadrupole cooling configurations.” B. Carlsten and K. Bishofberger, New J. Phys. 8 286 (2006).

- “Adapting Optics for High Energy Electron Cooling.” Ya. Derbenev, University of Michigan Report No. UM-HE-98-04, 1998.

Attachments